人抬化轻便钻机配套动力头的技术特点对照说明

The Comparative Explanation of Technical Characteristics of Several Matching Power swivels

| 参考照片 Reference photo |  |  |  |  |
| --- | --- | --- | --- | --- |
| 适用钻井方法 Applicable drilling methods | 泥浆/空气/螺旋钻井Mud scraping drilling Air scraping/DTH Hammer drilling Auger drilling | 泥浆/空气/螺旋钻井Mud scraping drilling Air scraping/DTH Hammer drilling Auger drilling | 泥浆/空气/螺旋钻井Mud scraping drilling Air scraping/DTH Hammer drilling Auger drilling | 空气/螺旋钻井 Air scraping/DTH Hammer drilling Auger drilling |
| 输出扭矩 Output torque (N.m) | 780/380 | 1100 | 1200 | 1200 |
| 输出转速 Output speed (r/min) | 0-140/300 | 0-220 | 0-160 | 0-160 |
| 外形尺寸 Overall dimensions (mm) | 313×299×622 | 283×260×625 | 191×210×497 | 191×210×491 |
| 重量(含马达等) Rig weight (kg) | 46.5 | 40.9 | 41.5 | 32.5 |
| 安装厚度 Mounting Thickness(mm) | 94 | 94 | 100 | 100 |
| 安装孔距 Mounting hole distance(mm) | 170 |  |  |  |
| 齿轮机械变速 Gear speed change | 是/双速 Yes /Dual speed | 是/单速 Yes/ Single speed | 否No | 否No |
| 导气/导水结构Air-conducting/water-conducting structure | 顶部轴向Top axial | 顶部轴向Top axial | 底部径向Bottom radial | 中部径向Middle radial |
| 自动补偿式密封圈Automatic compensating sealing ring | 是Yes | 是Yes | 否/手动调整 No/ Manual adjustment | 否/不可调整 No/ Unadjustable |
| 轴承/齿轮润滑方式Lubrication methods | 润滑油Oil | 润滑油Oil | 润滑脂Grease | 润滑脂Grease |
| 液压马达互换性Interchangeability of hydraulic motors | 完全互换 Completely interchangeable | 完全互换 Completely interchangeable | 完全互换 Completely interchangeable | 完全互换 Completely interchangeable |
| 下接头互换性Interchangeability of the lower adapter | 完全互换 Completely interchangeable | 完全互换 Completely interchangeable | 完全互换 Completely interchangeable | 完全互换 Completely interchangeable |